Gravity flow size exclusion purification of recombinant AAV vectors using Sepharose G100 SF resin

- Purify your recombinant AAV vector prep using the standard VVC Optiprep density gradient protocol to give you ~3 ml of purified vector in 1xGB plus Optiprep.

- Quantify your AAV prep by Q-PCR

- Store your AAV prep at 4 degrees

To prepare the Sepharose resin:

- Add 10g of Sepharose G100 SF powder to 250 ml of sterile filtered 1 x gradient buffer (1g of resin per 25ml buffer).

- Autoclave and cool to room temp. Keep overnight to allow time for the Sepharose beads to fully hydrate. Resin can be stored at room temp for ~6 months if the bottle is sealed with parafilm to prevent contamination

Loading the Columns:

- Take one 25ml Biorad Econopac column with upper bed support, lid and stopper per AAV to be purified. Spray all down with 70% ethanol and air dry in the tissue culture hood under UV light to sterilize.

- Rinse out the column with sterile filtered 1xGB to remove any excess ethanol

- Carefully load 25ml of the hydrated Sepharose G100SF resin into each column, avoiding the formation of air-bubbles in the column

- Add enough 1xGB to the top of the resin to fill up the column. Be careful not to disturb the top of the resin

- Allow all of the buffer to flow out of the column – will stop dripping when it is done

- Push the upper column bed support into the column and down onto the top of the resin

- Wash the resin with additional 1xGB – enough to fill up the column.

- Allow the wash buffer to flow through the resin until it stops dripping.

- Attach the yellow stopper to the column tip and add 1 ml of buffer to the upper reservoir. The column can now be capped, sealed with Parafilm and stored at 4 degrees until needed.

Purifying the AAV vectors:

- Set up 20 sterile 1.5 ml tubes per sample in a rack. Label with the vector name and label from 1 -20 from left to right. These will be used to collect fractions containing your virus and the flow-through from the column.

- Remove the stopper and allow excess buffer to drain

- Replace the stopper

- Add 3 ml of your Optiprep purified AAV vector to the top of the column

- Place the first collection tube under the column and remove the stopper

- Collect 1 ml fractions from the column until the drips stop

- Add 15ml of 1xGB to the top of the column and continue collecting 1 ml fractions.

- The early fractions should contain your AAV vector and later fractions will contain the Optiprep

- Take 50ul of each fraction collected and measure the absorbance at 260nm and 280nm using a spectrophotometer. Use 1xGB as the BLANK.

- Take all of the fractions from the start of collection up to the point where the absorbance begins to reach so high that the spec cannot measure it – these fractions contain your AAV vector WITHOUT Optiprep. Fractions above this point are where the Optiprep is being eluted – it has a very high absorbance at 260nm.

- Pool all of the virus containing fractions, bring the volume to 15ml with 1xGB and use a Vivaspin 15R 30 KD cut-off concentrator spin column to concentrate the sample down to 1 ml of less depending on the titer that is needed. Spin at 3,000 x g at 18 degrees C in the Beckman swinging bucket rotor.

- Re-titer the purified AAV vectors by Q-PCR to confirm recovery and yield.